AIW FAQ

AIW-172 driver & kernel compile on x86 platform

| Category | AIW | Date | 2026/04/09 |
| --- | --- | --- | --- |
| Keyword | AIW-172, driver, Ubuntu 20.04.6 LTS, Ubuntu 22.04.5 LTS, kernel 5.15.24+, MIO-5377R, ARK-1250L |  |  |

Category:

1.	Prerequisite	2

2.	Check Device	2

3.	Install Tool Packages	3

4.	Compile Kernel 5.15.24+	5

5.	Build WLAN Driver	8

6.	Load WLAN Driver	9

7.	Establish WLAN Connection	11

8.	Build and Load Bluetooth Driver	13

9.	Bluetooth test with Fluoride stack	15

- Prerequisite

MIO-5377R or ARK-1250L that has been installed Ubuntu 20.04.6 LTS or Ubuntu 22.04.5 LTS with normal installation.

- Check Device

- Check PCIe interface for WiFi

$ lspci

- Check USB interface for BT.

$ lsusb

- Install Tool Packages

[Ubuntu 20.04.6 LTS]

$ sudo apt-get update

$ sudo apt-get install iperf

$ sudo apt-get install iperf3

$ sudo apt-get install python

$ sudo apt-get install crash

$ sudo apt-get install ssh

$ sudo apt-get install net-tools

$ sudo apt-get install vim

$ sudo apt-get install -y openssh-server dos2unix wireless-tools exfat-utils exfat-fuse

$ sudo apt-get install linux-crashdump

Note that select “Yes” for both the prompts that show during “kexec-tools” and “kdump-tools” installation as below.

$ sudo apt-get install build-essential libncurses5-dev flex bison libnl-3-dev

$ sudo apt-get install kernel-package openssl

Note that select “Keep the local version currently installed” and press “Enter” to continue.

$ sudo apt-get install libssl-dev

$ sudo apt-get install libelf-dev

$ sudo apt-get install -y libnl-genl-3-dev

$ sudo apt-get install libdbus-1-dev

$ sudo apt-get install git

$ sudo apt-get install -y autoconf libtool

$ git config --global user.email "user email address"

$ git config --global user.name "user name"

$ sudo dpkg-reconfigure tzdata

[Ubuntu 22.04.5 LTS]

$ sudo apt-get update

$ sudo apt-get install iperf

$ sudo apt-get install iperf3

$ sudo apt-get install crash

$ sudo apt-get install ssh

$ sudo apt-get install net-tools

$ sudo apt-get install vim

$ sudo apt-get install -y openssh-server dos2unix wireless-tools exfatprogs exfat-fuse

$ sudo apt-get install linux-crashdump

Note that select “Yes” for both the prompts that show during “kexec-tools” and “kdump-tools” installation as below.

$ sudo apt-get install build-essential libncurses5-dev flex bison libnl-3-dev

$ sudo apt-get install openssl

$ sudo apt-get install libssl-dev

$ sudo apt-get install libelf-dev

$ sudo apt-get install -y libnl-genl-3-dev

$ sudo apt-get install libdbus-1-dev

$ sudo apt-get install git

$ sudo apt-get install -y autoconf libtool

$ git config --global user.email "user email address"

$ git config --global user.name "user name"

$ sudo dpkg-reconfigure tzdata

- Compile Kernel 5.15.24+

- Download Linux kernel version 5.15.24.

$ git clone git://git.kernel.org/pub/scm/linux/kernel/git/stable/linux-stable.git

$ cd linux-stable

$ git checkout v5.15.24

- Download Linux kernel version 5.15.24 patches.

$ git clone https://git.codelinaro.org/clo/sba-patches/wlan_patches.git

- Apply Linux kernel patches 0001-0179.

- Add the patch.sh file to the linux-stable folder.

$ cd /home/advantech/linux-stable/

$ sudo nano patch.sh

patch.sh:

#!/bin/bash
function patchfile ()
{
    for file in `ls $1`
    do
        if [ -d $1"/"$file ]
        then
            patchfile $1"/"$file
        else
            echo $1"/"$file
            git apply $1"/"$file
            echo `basename $file`
        fi
    done
}
patchfile $1

- Run “sudo chmod 777 patch.sh”.

$ sudo chmod 777 patch.sh

- Run “./patch.sh wlan_patches/fixce/3rdparty/patches/wlan_patches/kernel/v5.15_cologne/”.

$ ./patch.sh wlan_patches/fixce/3rdparty/patches/wlan_patches/kernel/v5.15_cologne/

- Copy kernel config file from the existing kernel 5.15.24 system to the kernel tree.

$ sudo cp wlan_patches/fixce/3rdparty/patches/wlan_patches/kernel/v5.15_cologne/config-5.15.24+ .config

- Make kernel config.

$ make menuconfig

load > ok > save > ok > exit > exit

- Build & install the Linux kernel.

$ make -j 2
$ make modules -j 2
$ sudo make modules_install
$ sudo make install

NOTE: make -j 2 -> depends on cpu cores

- Reboot with kernel 5.15.24+.

To make sure that the kernel version 5.15.24+ could be selected after system restart, edit grub setting as following screenshot then save it.

$ sudo nano /etc/default/grub

$ sudo update-grub

- After system restart, it will pop up grub menu during bootup process so that users could select kernel 5.15.24+ manually.

- Build WLAN Driver

- Download driver source package, unzip it and copy it to the system.

AIW_172_driver.zip

- Move to the folder that unzipped in last step and download driver codes with script. Here, I put “AIW_172_driver” under /home/advantech/Desktop/.

$ cd /home/advantech/Desktop/AIW_172_driver/AIO/apps/

$ sudo rm -r hostap/

$ cd /home/advantech/Desktop/AIW_172_driver/aio-gen/

$ sudo chmod 777 aio_gen_bit.sh
$ ./aio_gen_bit.sh -r -t cgn-f10

$ cd ../../

- Copy host folder to “/” and modify folder name from AIW_172_driver to 172.

$ sudo cp -fr ./AIW_172_driver /172

- Apply patches

$ cd /172/AIO/build/

$ sudo git config --global user.email "your_email"

$ sudo git config --global user.name "your_name"

$ sudo make drivers_patch

- Build the WLAN driver

$ sudo make

After driver built successfully, then there will be two driver files wlan_cnss_core_pcie.ko & wlan.ko under /172/AIO/rootfs-cgn-f10.build/lib/modules/.

- Load WLAN Driver

- Copy firmware from driver source package path to /lib/firmware/. (qcc2072-lea-1-0_qca_standard_oem-r00040.1-b592ec237fb2792b726af7b9905a3822ca65049b is in path /home/advantech/Desktop/.)

qcc2072-lea-1-0_qca_standard_oem-r00040.1-b592ec237fb2792b726af7b9905a3822ca65049b.zip

$ cd /home/advantech/Desktop/qcc2072-lea-1-0_qca_standard_oem-r00040.1-b592ec237fb2792b726af7b9905a3822ca65049b/meta_build/load_meta/wlan_firmware/

$ sudo cp -f ./* /lib/firmware/

- Copy qcom_cfg.ini file to /lib/firmware/wlan/. (qcom_cfg.ini is in path /home/advantech/Desktop/.)

qcom_cfg.ini

$ cd /home/advantech/Desktop/

$ sudo mkdir -p /lib/firmware/wlan/

$ sudo cp -f ./qcom_cfg.ini /lib/firmware/wlan/

- Edit qcom_cfg.ini file and confirm following options are included.

$ sudo nano /lib/firmware/wlan/qcom_cfg.ini

gDot11Mode=0 
gindoor_channel_support= 1 
scan_mode_6ghz=2

- Load WLAN drivers.

$ sudo modprobe cfg80211
$ cd /172/AIO/rootfs-cgn-f10.build/lib/modules/
$ sudo insmod ./wlan_cnss_core_pcie.ko
$ sudo insmod ./wlan.ko country_code=US

*country_code could be replaced accordingly

- Establish WLAN Connection

- Stop NetworkManager & wpa_supplicant service.

$ sudo systemctl stop NetworkManager.service

$ sudo systemctl stop wpa_supplicant.service

- Create wpa_supplicant.conf.

$ sudo nano /etc/wpa_supplicant/wpa.conf

- wpa.conf for 6GHz(WPA3):

ctrl_interface=/var/run/wpa_supplicant

update_config=1

ap_scan=1

fast_reauth=1

#6GHz_setting

pmf=1

sae_pwe=1

#6GHz_setting

network={

ssid="RF_6G"

psk="91779177"

scan_ssid=1

pairwise=CCMP TKIP

group=CCMP TKIP

proto=RSN

key_mgmt=SAE

ieee80211w=1

}

- wpa.conf for 2.4GHz/5GHz(WPA2):

ctrl_interface=/var/run/wpa_supplicant

update_config=1

network={

ssid="RF_5G"

psk="1234567890"

}

- Check WLAN interface name & run wpa_supplicant.

$ iw dev
$ sudo /172/AIO/rootfs-cgn-f10.build/sbin/wpa_supplicant -Dnl80211 -i wlan0 -c /etc/wpa_supplicant/wpa.conf -B

- Get IP address by DHCP and check network connection.

$ sudo dhclient wlan0

$ ping 8.8.8.8 -i wlan0

- Disconnect the current connected SSID.

$ sudo killall wpa_supplicant

- Build and Load Bluetooth Driver

qcc2072-lea-1-0_qca_standard_oem-r00040.1-b592ec237fb2792b726af7b9905a3822ca65049b.zip

- Build Bluetooth Driver

$ cd qcc2072-lea-1-0_qca_standard_oem-r00040.1-b592ec237fb2792b726af7b9905a3822ca65049b/chss_proc/host/
$ sudo chmod 777 aio-gen/build/scripts/cgn-f10/x86_bt.sh

$ BT_CONTROLLER_TYPE=gno_usb ./aio-gen/build/scripts/cgn-f10/x86_bt.sh dpb /home/advantech/Desktop/AIW_172_BT

- Load Bluetooth Driver

- Create folder.

$ sudo mkdir -p /lib/firmware/ar3k/

$ sudo mkdir -p /etc/bluetooth/

$ sudo mkdir -p /data/misc/bluetooth/

- Copy firmware, lib and configuration files.

$ sudo cp /home/advantech/Desktop/qcc2072-lea-1-0_qca_standard_oem-r00040.1-b592ec237fb2792b726af7b9905a3822ca65049b/meta_build/load_meta/bt_firmware/usb/* /lib/firmware/ar3k/
$ sudo cp -f /home/advantech/Desktop/AIW_172_BT/x86_64_out_rootfs/usr/lib/* /usr/lib/

$ sudo cp -f /home/advantech/Desktop/AIW_172_BT/x86_64_out_rootfs/etc/bluetooth/* /etc/Bluetooth/

- Remove BlueZ stack.

$ sudo apt-get remove bluez

$ sudo apt-get remove --auto-remove bluez

$ sudo apt-get purge bluez

$ sudo apt-get purge --auto-remove bluez

- Rename bluetooth folder under kernel path

$ sudo mv -f /lib/modules/5.15.24+/kernel/drivers/bluetooth/ /lib/modules/5.15.24+/kernel/drivers/bluetooth_ori/

- Load Bluetooth driver.

$ sudo insmod /home/advantech/Desktop/AIW_172_BT/x86_64_out_rootfs/bt_usb_qcom.ko

- Bluetooth test with Fluoride stack

To change the local Bluetooth device name, use the following command to edit the bt_app.conf file and specify the device name in the BtLocalDeviceName setting.

$ sudo nano /etc/bluetooth/bt_app.conf

- Enable BT and pair with BT devices

$ sudo su

$ /home/advantech/Desktop/AIW_172_BT/x86_64_out_rootfs/debug/btproperty &

$ /home/advantech/Desktop/AIW_172_BT/x86_64_out_rootfs/debug/btapp

$ gap_menu (go to gap menu)

$ enable (Enable BT)

$ inquiry (Scan device)

$ pair <bt_address> (pair bt device)

$ main_menu

- Test SPP(Serial Port Profile) server

* Prepare an Android phone and install “Serial Bluetooth Terminal” app

- Check bt_app.conf

BtSppServerEnable=true

BtSppClientEnable=true

- Start btproperty and btapp with root privilege

$ sudo su

$ /home/advantech/Desktop/AIW_172_BT/x86_64_out_rootfs/debug/btproperty &

$ /home/advantech/Desktop/AIW_172_BT/x86_64_out_rootfs/debug/btapp

- Enable BT and setup SPP service

$ gap_menu (go to gap menu)

$ enable (Enable BT)

$ inquiry (Scan device)

$ pair <bt_address> (pair bt device)

Go to main menu

$ main_menu

$ spp_server_menu

- Use Android phone and open “Serial Bluetooth Terminal” app for following operations.

- Select “Devices”.

- Refresh to show available BT devices and click DUT to make connection establish.

- Test receiving data.

- Setting on Android phone “Serial Bluetooth Terminal”:

- Command for receiving data on DUT.

- Test sending data:

- Command for sending data on DUT.

- Content of test.txt shown on Android phone “Serial Bluetooth Terminal”

- Test A2DP(Advanced Audio Distribution Profile) source

- Check bt_app.conf

BtA2dpSourceEnable=true

BtA2dpSinkEnable=false

BtAvrcpEnable=false

- Start btproperty and btapp with root privilege

$ sudo su

$ /home/advantech/Desktop/AIW_172_BT/x86_64_out_rootfs/debug/btproperty &

$ /home/advantech/Desktop/AIW_172_BT/x86_64_out_rootfs/debug/btapp

- Enable BT and start A2DP

$ gap_menu (go to gap menu)

$ enable (Enable BT)

$ inquiry (Scan device)

$ pair <bt_address> (pair bt device)

$ main_menu

$ a2dp_source_menu

$ connect <bt_address>

$ start

*Test with AirPods as BT client and play with following pcmtest.wav: pcmtest.wav

- Test BLE GATT(Generic Attribute Profile) server

* Prepare an Android phone and install “LightBlue” app

- Start btproperty and btapp with root privilege

$ sudo su

$ /home/advantech/Desktop/AIW_172_BT/x86_64_out_rootfs/debug/btproperty &

$ /home/advantech/Desktop/AIW_172_BT/x86_64_out_rootfs/debug/btapp

- Enable BT and start GATT

$ gap_menu (go to gap menu)

$ enable (Enable BT)

$ main_menu

$ gattstest_menu

$ gattstest_init

$ gattstest_start

- Use Android phone and open “LightBlue” app for following options.