AIW(EWM) FAQ

| Category | AIW | Date | 2022/09/23 |
| --- | --- | --- | --- |
| Keyword | AIW-163, driver, Ubuntu 20.04, kernel 5.4 |  |  |

- Question:

How to install AIW-163 driver on Ubuntu?

- Instructions :

- Download the driver package via below link.

RTL8852BE_WiFi_linux_v1.15.6.0.2-0-gac110bf5.20211029(55435).7z

- Unzip “RTL8852BE_WiFi_linux_v1.15.6.0.2-0-gac110bf5.20211029(55435).7z” using command

# 7z x RTL8852BE_WiFi_linux_v1.15.6.0.2-0-gac110bf5.20211029(55435).7z

- Access to “WiFi” folder.

- Decompress “RTL8852BE_WiFi_linux_v1.15.6.0.2-0-gac110bf5.20211029.tar.gz” using command

# tar zxvf RTL8852BE_WiFi_linux_v1.15.6.0.2-0-gac110bf5.20211029.tar.gz

- Access to “RTL8852BE_WiFi_linux_v1.15.6.0.2-0-gac110bf5.20211029” folder.

- Access to “driver” folder.

- Decompress “rtl8852BE_WiFi_linux_v1.15.6.0.2-0-gac110bf5.20211029.tar.gz”.

using command

# tar zxvf rtl8852BE_WiFi_linux_v1.15.6.0.2-0-gac110bf5.20211029.tar.gz

- Access to “rtl8852BE_WiFi_linux_v1.15.6.0.2-0-gac110bf5.20211029” folder.

- Input command “sudo make” to compile driver package to generate ko files.

If it’s ARM platform, please input command “sudo make ARCH=arm64” instead.

*If "make" error, try to use below command to fix:

# sudo apt install make

# sudo apt install build-essential

If still can’t make it work, try to copy the whole “rtl8852BE_WiFi_linux_v1.15.6.0.2-0-gac110bf5.20211029” folder to the other path, ex. desktop of PC then try to compile again.

After that, there’re a few files generated, including “8852be.ko”.

- Try to insert driver with below command.

# insmod 8852be.ko

*If error -2 issue(could not insert module XXX: Unknown symbol in module) happens while trying to insert driver, trying to use below command to fix:

# modprobe cfg80211

# insmod 8852be.ko again

- Finally, input below command to install the driver so that the driver will be loaded automatically in the future.

# make install

*Test environment: