AIW(EWM) FAQ

| Category | AIW | Date | 2026/02/04 |
| --- | --- | --- | --- |
| Keyword | AIW-163BR, EWM-W179M201E, STA+SAP concurrent, Ubuntu 22.04.5, kernel 6.8.0-90-generic, MIO-5377R |  |  |

Category:

1.	Compile hostapd to support 802.11ax	2

2.	Compile Wi-Fi driver	4

3.	Unload inbox rtw89 driver and load driver 8852be.ko manually	5

4.	Establish station connection	6

5.	SoftwareAP setup	8

- Compile hostapd to support 802.11ax

$ sudo apt update

$ sudo apt install build-essential

$ sudo apt install libnl-3-dev

$ sudo apt install libnl-genl-3-dev

$ sudo apt install pkg-config

$ sudo apt install libssl-dev

$ sudo apt install libdbus-1-dev

$ sudo apt install libreadline-dev

$ sudo apt install libnl-genl-3-dev

- Download hostapd-2.11 source code and decompress it then put it on Ubuntu system.

https://w1.fi/releases/

- Modify config and add make sure following configs are enabled.

$ cd hostapd-2.11/hostapd

$ cp defconfig .config

$ sudo nano .config

CONFIG_ACS=y

CONFIG_IEEE80211AC=y

CONFIG_IEEE80211AX=y

CONFIG_HE=y

CONFIG_SAE=y

CONFIG_SAE_PK=y

- Compile hostapd.

$ sudo make -j2
$ sudo make install

- Check hostapd version.

- Compile Wi-Fi driver

$ sudo apt update
$ sudo apt install make
$ sudo apt install build-essential
$ sudo apt install gcc-12 g++-12

- Download driver package and decompress then place it under /home/advantech/Desktop/.

rtl8852BE_WiFi_linux_v1.19.11-248-gf8ef8b7d8.20250221_sparklan_v1.0.tar.gz

- Edit Makefile and modify CONFIG_CONCURRENT_MODE option.

$ cd /home/advantech/Desktop/rtl8852BE_WiFi_linux_v1.19.11-248-gf8ef8b7d8.20250221_sparklan_v1.0/rtl8852BE_WiFi_linux_v1.19.11-248-gf8ef8b7d8.20250221/
$ sudo nano Makefile

CONFIG_CONCURRENT_MODE = y

- Compile Wi-Fi driver.

$ sudo make

After driver successfully compiled, there will be 8852be.ko appears

- Unload inbox rtw89 driver and load 8852be.ko manually

$ sudo rmmod rtw89_8852be
$ sudo rmmod rtw89_8852b
$ sudo rmmod rtw89_core
$ sudo rmmod rtw89_pci
$ lsmod | grep rtw89

- Load Wi-Fi driver.

$ cd /home/advantech/Desktop/rtl8852BE_WiFi_linux_v1.19.11-248-gf8ef8b7d8.20250221_sparklan_v1.0/rtl8852BE_WiFi_linux_v1.19.11-248-gf8ef8b7d8.20250221/
$ sudo insmod 8852be.ko rtw_country_code=TW

There will be two wireless interfaces after driver successfully loaded.

- Establish station connection

- Edit wpa_supplicant config.

$ sudo systemctl stop NetworkManager.service
$ sudo systemctl stop wpa_supplicant.service
$ sudo nano /home/advantech/Desktop/wpa.conf

wpa.conf:

pmf=2
sae_pwe=1
network={
        ssid="MLO-AP-433"
        key_mgmt=SAE
        ieee80211w=1
        sae_password="1234567890"
        pairwise=CCMP
}

- *Above wpa_supplicant config is for WPA3-SAE SSID. Please refer to following wpa_supplicant config of WPA2-PSK.

pmf=1
network={
        scan_ssid=1
        ssid="RF_5G"
        psk="1234567890"
        proto=RSN
        key_mgmt=WPA-PSK
        pairwise=CCMP
}

- Execute wpa_supplicant to establish Wi-Fi connection.

$ sudo wpa_supplicant -Dnl80211 -i wlan1 -c /home/advantech/Desktop/wpa.conf

- Request DHCP IP.

$ sudo dhclient wlan1

$ ifconfig

wpa_supplicant version:

- SoftwareAP setup

- Edit hostapd config.

$ sudo nano /home/advantech/Desktop/hostapd.conf

hostapd.conf:

ctrl_interface=/var/run/hostapd
interface=wlp43s0
driver=nl80211

ctrl_interface_group=0
ssid=_8852_aptest
hw_mode=a
channel=36

wpa_passphrase=1234567890
wpa=2
wpa_key_mgmt=WPA-PSK
rsn_pairwise=CCMP

ieee80211n=1
ht_capab=[HT40+][HT40-][SHORT-GI-20][SHORT-GI-40]
ieee80211ac=1
vht_oper_chwidth=1
vht_oper_centr_freq_seg0_idx=42
ieee80211ax=1
he_oper_chwidth=1
he_oper_centr_freq_seg0_idx=42
country_code=TW

- Edit DHCP server.

$ sudo apt install isc-dhcp-server
$ sudo nano /etc/default/isc-dhcp-server

INTERFACESv4=”wlp43s0”

- Assign DHCP clients IP range.

$ sudo nano /etc/dhcp/dhcpd.conf

dhcpd.conf:

subnet 10.10.0.0 netmask 255.255.255.0 {
        range 10.10.0.2 10.10.0.16;
        option domain-name-servers 8.8.4.4, 8.8.8.8;
        option routers 10.10.0.1;
}

- Specify static IP address for the wireless interface of softwareAP.

$ sudo ifconfig wlp43s0 10.10.0.1

- Start isc-dhcp-server service.

$ sudo service isc-dhcp-server start
$ sudo systemctl status isc-dhcp-server

- Execute hostapd to start softwareAP.

$ sudo hostapd-2.11/hostapd/hostapd -dd /home/advantech/Desktop/hostapd.conf

Ubuntu 22.04.5 LTS, kernel 6.8.0-90-generic