How to Setup Network with AIW-341

Prerequisite

- EI-53 and setup Ubuntu 24.04.1 (kernel 6.8.0) on it.

Check Device

Before driver installed, you will see the device as below by “lsusb”.

$ lsusb Bus 004 Device 002: ID 1bc7:1060 Telit Wireless Solutions LN920

Install Driver

# sudo su # modprobe option # echo 1bc7 <pid> > /sys/bus/usbserial/drivers/option1/new_id

You can get the pid by lsusb command.

Example:

You got the lsusb return like below:

Bus 004 Device 002: ID 1bc7:1060 Telit Wireless Solutions LN920

The pid is “1060”, then the command will be “echo 1bc7 1060 > /sys/bus/usbserial/drivers/option1/new_id”

Check Device After installed driver

Please check com port device by “ls /dev/ttyUSB*”

# ls /dev/ttyUSB* ttyUSB0 ttyUSB1 ttyUSB2 ttyUSB3

Please check network device by “ls /dev/cdc-wdm*”

# ls /dev/cdc-wdm* cdc-wdm3

Function Test

- Access AT port

- Check the number of COM port with AIW-341 by “ls /dev/ttyUSB*”.

- Access to AT command through terminal like putty/ minicom.

Please make setting as below:

  - Baud Rate “115200”

  - Data bits "8"

  - Stop bits "1"

  - Parity "None"

  - Flow Control "None"

- Set USB configuration on the modem device.

  - at#usbcfg

    - Set USB to RMnet

      - at#usbcfg=1

    - Set USB to MBIM

      - at#usbcfg=2

Example: at#usbcfg? #USBCFG: 1 OK

- list of PDP addresses for the specified context identifiers.

  - at+cgpaddr=<cid>

Example: at+cgpaddr=1 +CGPADDR: 1,"10.131.111.42","32.1.180.0.227.218.248.48.80.36.142.70.93.113.197.2"

- Reports the device board serial number.

  - at+gsn

Example: at+gsn 352214163199574

- Selects a network operator and registers the module.

  - at+cops?

Example: at+cops? +COPS: 0,0,"Chunghwa Telecom",7

- Reads current network status.

  - at#rfsts

at#rfsts #RFSTS: "466 92",1400,-98,-72,-9,3391,255,22,640,3,2,4F65A0C,"466924103488171","Chunghwa Telecom",3,3

- Specifies the SIM detection mode.

  - AT#SIMDET=<mode>

at#simdet? #SIMDET: 0,1

- Selects RF bands

  - AT#BND=<GSM_band>[,<UMTS_band>[,<LTE_band>[,<LTE_band_ext>]]]

- Set LTE bnad 1 and bnad 17

Example: AT#BND? #BND: 0,0,10001,0

- Unlock pin

At+cpin=<pin>

mbimcli -p -d /dev/cdc-wdmX --enter-pin=####

- GNSS test

Please follow the below steps to test GNSS.

Noted that please use passive antenna on AIW-341.

- Enable GNSS by at command

# AT$GPSP=1

# AT$GPSNMUN=1,0,0,1,0,0,0

- Connect to nmea port, then you will see the GNSS log.

- Connection test

  - Use RMnet to connect to internet.

$ sudo vi /etc/qmi-network.conf APN=internet APN_USER= APN_PASS= PROXY=yes

$ sudo qmi-network /dev/cdc-wdm3 start Loading profile at /etc/qmi-network.conf... APN: internet APN user: unset APN password: unset qmi-proxy: yes IP_TYPE: unset PROFILE: unset Checking data format with 'qmicli -d /dev/cdc-wdm3 --wda-get-data-format --device-open-proxy'... Device link layer protocol retrieved: raw-ip Getting expected data format with 'qmicli -d /dev/cdc-wdm3 --get-expected-data-format'... Expected link layer protocol retrieved: raw-ip Device and kernel link layer protocol match: raw-ip Starting network with 'qmicli -d /dev/cdc-wdm3 --wds-start-network=apn='internet' --client-no-release-cid --device-open-proxy'... Saving state at /tmp/qmi-network-state-cdc-wdm3... (CID: 20) Saving state at /tmp/qmi-network-state-cdc-wdm3... (PDH: 1164086512) Network started successfully

- Use MBIM to connect to internet

$ sudo vi /etc/mbim-network.conf APN=internet APN_USER= APN_PASS= PROXY=yes

$ sudo mbim-network /dev/cdc-wdm3 start Loading profile at /etc/mbim-network.conf... APN: internet APN auth protocol: unset APN user: unset APN password: unset mbim-proxy: yes Querying subscriber ready status 'mbimcli -d /dev/cdc-wdm3 --query-subscriber-ready-status --device-open-proxy'... [/dev/cdc-wdm3] Subscriber ready status retrieved: Ready state: 'initialized' Subscriber ID: '466924103488171' SIM ICCID: '89886920041034881711' Ready info: 'none' Telephone numbers: (0) 'unknown' Flags: 'sim-removable' Querying registration state 'mbimcli -d /dev/cdc-wdm3 --query-registration-state --device-open-proxy'... [/dev/cdc-wdm3] Registration status: Network error: 'none' Register state: 'home' Register mode: 'automatic' Available data classes: 'lte' Current cellular class: 'gsm' Provider ID: '46692' Provider name: 'Chunghwa Telecom' Roaming text: 'unknown' Registration flags: 'packet-service-automatic-attach' Preferred data classes: 'umts, hsdpa, hsupa, lte' Attaching to packet service with 'mbimcli -d /dev/cdc-wdm3 --attach-packet-service --device-open-proxy'... [/dev/cdc-wdm3] Successfully attached to packet service [/dev/cdc-wdm3] Packet service status: Network error: 'none' Packet service state: 'attached' Available data classes: 'lte' Uplink speed: '50000000 bps' Downlink speed: '100000000 bps' Frequency range: 'unknown' Data sub class: 'none' TAI PLMN MCC: '000' TAI PLMN MNC: '000' TAI TAC: '0' Starting network with 'mbimcli -d /dev/cdc-wdm3 --connect=apn='internet' --device-open-proxy'... warning: key 'apn' is deprecated, use 'access-string' instead Network started successfully ubuntu@ubuntu:~$ sudo udhcpc -i wwan0 udhcpc: started, v1.36.1 Dropped protocol specifier '.udhcpc' from 'wwan0.udhcpc'. Using 'wwan0' (ifindex=4). udhcpc: broadcasting discover udhcpc: broadcasting select for 25.103.116.33, server 25.103.116.34 udhcpc: lease of 25.103.116.33 obtained from 25.103.116.34, lease time 7200 Dropped protocol specifier '.udhcpc' from 'wwan0.udhcpc'. Using 'wwan0' (ifindex=4).

  - Change wwan to use Raw-IP.

$ ip link set dev wwan0 down $ echo Y > /sys/class/net/wwan0/qmi/raw_ip $ ip link set dev wwan0 up

- Use udhcpc or dhclient to get ip address

$ sudo udhcpc -i wwan0 udhcpc: started, v1.36.1 Dropped protocol specifier '.udhcpc' from 'wwan0.udhcpc'. Using 'wwan0' (ifindex=4). udhcpc: broadcasting discover udhcpc: broadcasting select for 25.103.116.33, server 25.103.116.34 udhcpc: lease of 25.103.116.33 obtained from 25.103.116.34, lease time 7200 Dropped protocol specifier '.udhcpc' from 'wwan0.udhcpc'. Using 'wwan0' (ifindex=4).