How to Setup Network with AIW-341

Prerequisite

- ARK-1251 and setup Windows 11 on it

Please refer to the below picture for checking the jumper setting of M2_SEL1 pin1-2 to NC to support USB interface on ARK-1251.

Check Device

Before driver installed, you will see the snapshot as below.

Install Driver

Check Device After installed driver

Function Test

- Access AT port

- Check the number of COM port with AIW-341 in device manager.

- Access to AT command through terminal like putty.

Please make setting as below:

  - Baud Rate “115200”

  - Data bits "8"

  - Stop bits "1"

  - Parity "None"

  - Flow Control "None"

- Set USB configuration on the modem device.

  - at#usbcfg

    - Set USB to RMnet

      - at#usbcfg=1

    - Set USB to MBIM

      - at#usbcfg=2

Example: at#usbcfg? #USBCFG: 1 OK

- list of PDP addresses for the specified context identifiers.

  - at+cgpaddr=<cid>

Example: at+cgpaddr=1 +CGPADDR: 1,"10.131.111.42","32.1.180.0.227.218.248.48.80.36.142.70.93.113.197.2"

- Reports the device board serial number.

  - at+gsn

Example: at+gsn 352214163199574

- Selects a network operator and registers the module.

  - at+cops?

Example: at+cops? +COPS: 0,0,"Chunghwa Telecom",7

- Reads current network status.

  - at#rfsts

at#rfsts #RFSTS: "466 92",1400,-98,-72,-9,3391,255,22,640,3,2,4F65A0C,"466924103488171","Chunghwa Telecom",3,3

- Specifies the SIM detection mode.

  - AT#SIMDET=<mode>

at#simdet? #SIMDET: 0,1

- Selects RF bands

  - AT#BND=<GSM_band>[,<UMTS_band>[,<LTE_band>[,<LTE_band_ext>]]]

- Set LTE bnad 1 and bnad 17

Example: AT#BND? #BND: 0,0,10001,0

- GNSS test

Please follow the below steps to test GNSS.

Noted that please use passive antenna on AIW-341.

- Enable GNSS by at command

# AT$GPSP=1

# AT$GPSNMUN=1,0,0,1,0,0,0

- Connect to nmea port, then you will see the GNSS log.