EWM FAQ

| Category | EWM | Date | 2018/03/22 |
| --- | --- | --- | --- |
| Keyword | EWM-C117FL0XE ,4G/LTE Connection |  |  |

- Question:

How to establish ppp connection on windows 7 OS?

- Instructions :

Step 1

Before set up dial up connection, please check module current connection mode in “m-center” utility for sure

Step 2

Download m-center utility by attached file

Step 3

Execute m-center utility to select specific com port

Step 5

After selecting Com port, press connect button for connecting, then press “Get info” button to acquire modem information

Step 6

Input SIM card’s PIN Code

Step 7

Switch to AT terminal mode and select specific AT group (ppp mode) to insert Commands orderly

Step 8

Close m-center utility

Step 9

Create a new connection and set dial up connect in network center

Step 10

Input dail up number in specific field

Step 11

Select other at the right side of dial up number’s input field

Step 12

Add “*99***1#” in number list

*Notice*: please remember to tick the option “if the current dial up number is failure ,please try to next number”

Step 13

Try to connect this connection directly for sure

Step 14

Download third party auto dial up tool below and set up the activate parameter by yourself https://www.dropbox.com/s/g2biiwj2gryutbk/%E7%AC%AC%E4%B8%89%E6%96%B9autodialup%20program.rar?dl=0