How to Setup GPS with AIW-210

Prerequisite

- UNO-137 and setup Windows 10 on it

- SSD with a storage capacity of at least 64GB

Install Utility

Please run the installer “u-center_v2x.xx.exe” and follow the prompt to install ubox u-center.

https://content.u-blox.com/sites/default/files/2024-11/u-centersetup_v24.10.zip

Check Serial Port

The serial port is present with driver free in “Device Manager”.

Get data by u-center

- Open u-center via “All Programs > u-blox > u-center > u-center” and you will see the snapshot like below.

- Select the serial port

Locate the communication toolbar and click on the arrow as the below icon. This will show a list with all available COM ports. Select the corresponding COM port where the receiver is connected.

- Set baud rate with serial port

In the communication toolbar, click on the below icon. This will show a

list with all available baud rates. Select the baud rate to 38400 on which the receiver is communicating. If u-center is able to decode data from the receiver, the status bar begins to blink in green. This means that the connection is established successfully and the communication between the receiver and u-center is working. Now you are ready to use the receiver.

- Show message on text console

In standard menu bar, click on the “View” menu. This will show a

list with menu entries. Select “Text Console” entry like below.

- Display message

Note that please setup attena in outdoor, it will show messages as below snapshoot.

More Information

Please refer to u-center user guide for more information if needed. The user can find out the user guide via “All Programs > u-blox > User Guide” and you will see the snapshot like below.