AIW FAQ

How to bring up AIW-172 Wifi feature with RSB-3720

| Category | AIW | Date | 2026/4/19 |
| --- | --- | --- | --- |
| Keyword | AIW-172(WiFi 7),RSB-3720(Yocto 5.2 LTS) with linux kernel 6.6.23) |  |  |

Catergory:

1.	Check Device	2

2.	Build Environment on Host	2

3.	Getting Linux Code and Setting Environment	2

4.	Build Kernel Image and build Tool-Chain	5

5.	Build WLAN Driver and network tool(hostapd and wpa_supplicant)	7

6.	Flash Image	11

7.	Install Driver	12

8.	Check Device After installed driver	12

Appendix:	12

- Check Device

Before driver installed, you will see the device as below by “lspci”.

$ lspci 00:00.0 PCI bridge: Synopsys, Inc. DWC_usb3 / PCIe bridge (rev 01) 01:00.0 Network controller: Qualcomm Technologies, Inc Device 1112 (rev 01)

- Build Environment on Host

[RSB-3720]

https://ess-wiki.advantech.com.tw/view/IoTGateway/BSP/Linux/iMX8/Yocto_LBVF_User_Guide

- Getting Linux Code and Setting Environment

[RSB-3720]

$ repo init -u git://github.com/ADVANTECH-Corp/adv-arm-yocto-bsp.git -b imx-linux-kirkstone -m imx8LBVF0071.xml

$ repo sync

$ MACHINE=imx8mprsb3720a2 UBOOT_CONFIG=6G DISTRO=fsl-imx-xwayland source imx-setup-release.sh -b build_x11

$ bitbake virtual/kernel

Copy the 0001-AIW-172-fix.patch to below path .

Download below file and untar it.

$ mkdir /home/adv/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/files

$ mv 0001-AIW-172-fix.patch /home/adv/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/files

Modify the .bbappend file

$ cd  /home/adv/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux

$ vi linux-imx_6.6.bbappend

Add below content in the end of the .bbappend file.

FILESEXTRAPATHS:prepend := "${THISDIR}/files:" SRC_URI += "file://0001-AIW-172-fix.patch"

$ vi /home/adv/adv-release-bsp/build_Yocto5/conf/local.conf

Add below content in the end of this file.

IMAGE_INSTALL:append = " iperf3" IMAGE_INSTALL:append=" dnsmasq linux-firmware-qca"

- Build Kernel Image and build Tool-Chain

[RSB-3720]

$ vi /home/adv/adv-release-bsp/sources/meta-imx/meta-imx-ml/recipes-nnstreamer/nnshark/nnshark_2023.11.imx.bb

Modify

NNSHARK_SRC ?= "gitsm://github.com/nxp-imx/nnshark.git;protocol=https"

to

NNSHARK_SRC ?= "git://github.com/nxp-imx/nnshark.git;protocol=https"

add below content in the nnshardk_2023.11.imx.bb

do_configure[network]="1"

do_configure:prepend() {

cd ${S}

echo ">>> Fixing GStreamer common submodule URL"

if grep -q "anongit.freedesktop.org" .gitmodules; then

sed -i 's#git://anongit.freedesktop.org/gstreamer/common#https://gitlab.freedesktop.org/gstreamer/common.git#g' .gitmodules || true

fi

git submodule sync --recursive || true

git submodule update --init --recursive || true

echo ">>> Submodule fix done."

}

$ vi ~/adv-release-bsp/sources/meta-imx/meta-imx-ml/recipes-libraries/onnxruntime/onnxruntime_1.17.1.bb

Add below function as below

DEPENDS = "libpng zlibi libeigen"

EXTRA_OECMAKE += "\

-DFETCHCONTENT_FULLY_DISCONNECTED=OFF \

-DCMAKE_BUILD_TYPE=RelWithDebInfo \

-Donnxruntime_BUILD_UNIT_TESTS=ON \

-Donnxruntime_USE_PREINSTALLED_EIGEN=ON\

-Deigen_SOURCE_PATH=${STAGING_INCDIR}/eigen3\

"

$ cd ~/adv-release-bsp/build_x11

$ bitbake imx-image-full

$ vi /home/adv/adv-release-bsp/build_Yocto5/conf/local.conf

Add below content in the end of the file

TOOLCHAIN_TARGET_TASK:append = " \ glibc \ glibc-dev \ libbsd \ libbsd-dev \ libnl \ libnl-dev \ libnl-genl \ openssl \

$ bitbake meta-toolchain

The below installer will be located in the directory "./tmp/deploy/sdk".

fsl-imx-wayland-glibc-x86_64-meta-toolchain-armv8a-imx8mp-lpddr4-evk-toolchain-${KERNEL}-scarthgap.sh

You need install the tool chain and source the environment

The full image will in the “./tmp/deploy/img”

- Build WLAN Driver and network tool(hostapd and wpa_supplicant)

5.1 Build WLAN Driver

- Download the driver source code from git server

[RSB-3720]

AIW_172_driver.zip

$ cd / $ sudo git clone ssh://adv_ae@172.22.24.155:/volume1/git-server/AIW_172_driver $ sudo mv /AIW_172_driver /172 $ cd /172

NOTE: the password for adv_ae is “123456”

- Build driver

$ vim /172/AIO/build/scripts/cgn-f10/config.cgn-f10 Modify below information in the config.cgn-f10 export KERNELPATH=/home/adv/adv-release-bsp/build_Yocto5/tmp/work/imx8mprsb3720a2-poky-linux/linux-imx/6.6.23+git/build export TOOLCHAIN=/opt/fsl-imx-xwayland/6.6-scarthgap/sysroots/x86_64-pokysdk-linux/usr/bin/aarch64-poky-linux/aarch64-poky-linux- export KERNELARCH=arm64 export CROSS_COMPILE=${TOOLCHAIN} export TOOLPREFIX=${CROSS_COMPILE}

Edit wpa_supplicant config

$ cd /172/AIO/drivers/patches/cgn-f10/supplicant $vi wpa_supplicant.config Add below content in the end of the file CFLAGS += --sysroot=$(SDKTARGETSYSROOT) LDFLAGS += --sysroot=$(SDKTARGETSYSROOT) CFLAGS += -I$(SDKTARGETSYSROOT)/usr/include/libnl3 CFLAGS += -I$(SDKTARGETSYSROOT)/usr/include/dbus-1.0 CFLAGS += -I$(SDKTARGETSYSROOT)/usr/lib/dbus-1.0/include LDFLAGS += -L$(SDKTARGETSYSROOT)/usr/lib -ldbus-1

Edit hostapd config

$ vi hostapd.config Add below content in the end of the file CFLAGS += --sysroot=$(SDKTARGETSYSROOT) LDFLAGS += --sysroot=$(SDKTARGETSYSROOT) CFLAGS += -I$(SDKTARGETSYSROOT)/usr/include/libnl3 CFLAGS += -I$(SDKTARGETSYSROOT)/usr/include/dbus-1.0 CFLAGS += -I$(SDKTARGETSYSROOT)/usr/lib/dbus-1.0/include LDFLAGS += -L$(SDKTARGETSYSROOT)/usr/lib -ldbus-1 $ cd /172/AIO/build $ make

- Get wlan_cnss_core_pcie.ko, wlan.ko from specific folder ( /172/AIO/rootfs-cgn-f10.build /lib/modules)

- The FW will in the specific folder

(/172/AIO/rootfs-cgn-f10.build/lib/firmware/WLAN-firmware)

  - hostapd and wpa_supplicant

- After build the driver, wpa_supplicant and hostapd will in the below path

/172/AIO/rootfs-cgn-f10.build/sbin

After built hostapd and wpa_supplicant, you need copy it to the SD card and boot up the RSB-3720. You need confirm “wpa_supplicant” version if is 2.11 version

- Flash Image

[RSB-3720]

Flash image to SD card

OS image path “/home/adv/adv-release-bsp/build_x11/tmp/deploy/images/imx8mprsb3720a2”

.wic.zst if full OS image

You need copy it to your system.

$ maker p ~/adv-release-bsp/Temp/SD_img

$ cp imx-image-full-imx8mprsb3720a2-20251209023921.rootfs.wic.zst ~/adv-release-bsp/Temp/SD_img

Change to your real system(Not in the docker)

- $ cd ~/Downloads/adv-release-bsp/Temp/SD_img

- $ zstd -d imx-image-full-imx8mprsb3720a1-xxxx.rootfs.wic.zst (Example: imx-image-full-imx8mprsb3720a2-20251209023921.rootfs.wic.zst)

- $ sudo dd if=imx-image-full-imx8mprsb3720a1-xxxx.rootfs.wic of=/dev/sdb bs=4M status=progress conv=fsync

- Install Driver

Copy firmware files to device: $sudo cp -rf /172/AIO/rootfs-cgn-f10.build/lib/firmware/WLAN-firmware/* /lib/firmware/ Insmod driver: $ sudo insmod <path to .ko>/wlan_cnss_core_pcie.ko $ sudo insmod <path to .ko>/wlan.ko country_code=US

- Check Device After installed driver

# ifconfig -a # ifconfig wlan0 up The result should like below: