AIW FAQ

How to bring up AIW-173 BlueTooth feature with RSB-3720

| Category | AIW | Date | 2025/12/9 |
| --- | --- | --- | --- |
| Keyword | AIW-173(WiFi 7),RSB-3720(Yocto 5.2 LTS) with linux kernel 6.6.23) |  |  |

Catergory:

1.	Check Device	2

2.	Build Environment on Host	2

3.	Getting Linux Code and Setting Environment	2

4.	Build Kernel Image and build Tool-Chain	5

5.	Flash Image	10

6.	Install Driver	11

Appendix: File transfer via BT	12

- Check Device

Before driver installed, you will see the device as below by “lspci”.

$ lspci 01:00.0 Network controller: Qualcomm Technologies, Inc WCN785x Wi-Fi 7(802.11be) 320MHz 2x2 [FastConnect 7800] (rev 01)

- Build Environment on Host

[RSB-3720]

https://ess-wiki.advantech.com.tw/view/IoTGateway/BSP/Linux/iMX8/Yocto_LBVF_User_Guide

- Getting Linux Code and Setting Environment

[RSB-3720]

$ repo init -u git://github.com/ADVANTECH-Corp/adv-arm-yocto-bsp.git -b imx-linux-kirkstone -m imx8LBVF0071.xml

$ repo sync

$ MACHINE=imx8mprsb3720a2 UBOOT_CONFIG=6G DISTRO=fsl-imx-xwayland source imx-setup-release.sh -b build_x11

$ bitbake virtual/kernel

$ mkdir -p ~/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/files

Move below file in the “/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/files”

$ mkdir -p ~/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/linux-imx/

Move below file in the “~/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/linux-imx/”

$ mv fragment.cfg ~/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/linux-imx/

$ nano ~/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/linux-imx_6.6.bbappend

Add content below at the end of this file.

FILESEXTRAPATHS:prepend := "${THISDIR}/files:"

SRC_URI += "file://0001-kernel-mods-for-6.6.23.patch"

KCONFIG_MODE = "merge"

SRC_URI += "file://fragment.cfg"

do_configure:append () {

cat ${WORKDIR}/*.cfg >> ${B}/.config

}

For enable Bluetooth test, you will need add below tool in the local config

$ cd /home/adv/adv-release-bsp/build_x11/conf

$ vi local.conf

Add below content at the end of “local.conf”

DISTRO_FEATURES:append = “ bluetooth”

IMAGE_INSTALL:append = ” bluez5 bluez5-noinst-tools bluez5-obex dbus-wait”

PACKAGECONFIG:append:pn-bluez5 = “ obex-profiles”

# Enable UTF-8 locale support

IMAGE_LINGUAS = " en-us zh-tw"

GLIBC_GENERATE_LOCALES = " en_US.UTF-8 zh_TW.UTF-8"

IMAGE_INSTALL:append = " \

glibc-utils \

locale-base-en-us \

locale-base-zh-tw \

glibc-gconvs \

glibc-gconv-utf-16 \

"

Rebuild kernel

$ bitbake -c cleansstate virtual/kernel

$ bitbake virtual/kernel

- Build Kernel Image and build Tool-Chain

[RSB-3720]

$ vi /home/adv/adv-release-bsp/sources/meta-imx/meta-imx-ml/recipes-nnstreamer/nnshark/nnshark_2023.11.imx.bb

Modify

NNSHARK_SRC ?= "gitsm://github.com/nxp-imx/nnshark.git;protocol=https"

to

NNSHARK_SRC ?= "git://github.com/nxp-imx/nnshark.git;protocol=https"

add below content in the nnshardk_2023.11.imx.bb

do_configure[network]="1"

do_configure:prepend() {

cd ${S}

echo ">>> Fixing GStreamer common submodule URL"

if grep -q "anongit.freedesktop.org" .gitmodules; then

sed -i 's#git://anongit.freedesktop.org/gstreamer/common#https://gitlab.freedesktop.org/gstreamer/common.git#g' .gitmodules || true

fi

git submodule sync --recursive || true

git submodule update --init --recursive || true

echo ">>> Submodule fix done."

}

$ vi ~/adv-release-bsp/sources/meta-imx/meta-imx-ml/recipes-libraries/onnxruntime/onnxruntime_1.17.1.bb

Add below function as below

DEPENDS = "libpng zlibi libeigen"

EXTRA_OECMAKE += "\

-DFETCHCONTENT_FULLY_DISCONNECTED=OFF \

-DCMAKE_BUILD_TYPE=RelWithDebInfo \

-Donnxruntime_BUILD_UNIT_TESTS=ON \

-Donnxruntime_USE_PREINSTALLED_EIGEN=ON\

-Deigen_SOURCE_PATH=${STAGING_INCDIR}/eigen3\

"

$ cd ~/adv-release-bsp/build_x11

$ bitbake imx-image-full

The full image will in the “./tmp/deploy/img”

- Flash Image

[RSB-3720]

Flash image to SD card

OS image path “/home/adv/adv-release-bsp/build_x11/tmp/deploy/images/imx8mprsb3720a2”

.wic.zst if full OS image

You need copy it to your system.

$ maker p ~/adv-release-bsp/Temp/SD_img

$ cp imx-image-full-imx8mprsb3720a2-20251209023921.rootfs.wic.zst ~/adv-release-bsp/Temp/SD_img

Change to your real system(Not in the docker)

- $ cd ~/Downloads/adv-release-bsp/Temp/SD_img

- $ zstd -d imx-image-full-imx8mprsb3720a1-xxxx.rootfs.wic.zst (Example: imx-image-full-imx8mprsb3720a2-20251209023921.rootfs.wic.zst)

- $ sudo dd if=imx-image-full-imx8mprsb3720a1-xxxx.rootfs.wic of=/dev/sdb bs=4M status=progress conv=fsync

- Install Driver and firmware

Download the driver source code from git server

[RSB-3720]

AIW_173_driver.tar.gz

$ cd / $ sudo git clone ssh://adv_ae@172.22.24.155:/volume1/git-server/AIW_173_driver $ sudo mv /AIW_173_driver /173 $ cd /173/

NOTE: the password for adv_ae is “123456”

Copy firmware files to device: $ sudo cp /173/AIO/build/bt_firmware/usb/* /lib/firmware/qca/

Appendix: File transfer via BT

RSB-3720(Yocto 5.0) transfer file to laptop(Windows)
$ /usr/libexec/bluetooth/obexd -r /tmp -a -n -d&

$ bluetoothctl

[bluetooth]# power on

[bluetooth]# agent on

[bluetooth]# discoverable on

[bluetooth]# scan on

check your bluetooth mac in the list

[bluetooth]# pair {your bluetooth mac}

[bluetooth]# scan off

[bluetooth]# exit

In the Windows side open receive file via bluetooth

When your windows system shows

Go back to RSB-3720 and issue below command

$ obexctl

[obex]# connect {your bluetooth mac}

[obex]# send {file name}

In the windows system, you will see the progress bar shows as below

[obex]# disconnect

In the windows system, you need select store path

You will see the file in the windows Documents folder

laptop(Windows) transfer file to RSB-3720(Yocto 5.0).

In the RSB-3720, you need issue below command at beginning.

$ /usr/libexec/bluetooth/obexd -r /tmp -a -n -d&

$ bluetoothctl

[bluetooth]# power on

[bluetooth]# agent on

[bluetooth]# discoverable on

[bluetooth]# scan on

check your bluetooth mac in the list or use #devices to check the devices mac

[bluetooth]# pair {your bluetooth mac}

[bluetooth]# scan off

[bluetooth]# exit

Select transfer file

Check the file in the RSB-3720

bluetoothctl

Obexd