AIW FAQ

How to bring up AIW-173 Wifi feature with AFE-R750/AIR-030

| Category | AIW | Date | 2025/11/11 |
| --- | --- | --- | --- |
| Keyword | AIW-173(WiFi 7),AIR-030(Nvidia Jetson Orin+JetPack 6.0 R36.3(Ubuntu OS 22.04.4 LTS) with linux kernel 5.15.136-tegra),AFE-R750(NvidiaJetson Orin+JetPack 6.1 R36.4(Ubuntu 22.04) with linux kernel 5.15.148) |  |  |

Catergory:

1.	Check Device	2

2.	Build Environment on Host	2

3.	Getting Linux Code and Setting Environment	3

4.	Build Kernel Image	4

5.	Build WLAN Driver	5

6.	Flash Image	6

7.	Build hostapd and wpa_supplicant	8

8.	Install Driver	9

9.	Check Device After installed driver	10

- Check Device

Before driver installed, you will see the device as below by “lspci”.

$ lspci 01:00.0 Network controller: Qualcomm Technologies, Inc WCN785x Wi-Fi 7(802.11be) 320MHz 2x2 [FastConnect 7800] (rev 01)

- Build Environment on Host

[AIR-030]

https://ess-wiki.advantech.com.tw/view/AIR-030

[AFE-R750]

Currently, we adopt Docker as build environment.
You can get the latest version of advrisc/u20.04-nvlbv2:20240515 Docker image for developing nVidia jetson projects.
Docker command:

$ sudo docker pull advrisc/u20.04-nvlbv2:20240515

Run Docker example

Step1 Run docker container (example)

$ sudo docker run -it --name jetson_linux_risc -v /home/bsp/myLinux:/home/adv/BSP:rw --privileged advrisc/u20.04-nvlbv2:20240515 /bin/bash

Step2 In container, change the user. (example)

$ sudo chown adv:adv -R BSP

- Getting Linux Code and Setting Environment

[AIR-030]

$ repo init -u https://AIM-Linux@dev.azure.com/AIM-Linux/risc_nvidia_jetson_36.3/_git/manifest -m air030.xml

$ repo sync

$ cd /home/adv/BSP/jetson_linux_risc/kernel

- Add AIW-173’s kernel patch to original kernel source

**Note** 
this specific patch file was detached from AFE-R750’s kernel source by using following command (git format-patch -100)

- Input “sudo patch -p1 --dry-run < /path/to/your/patchfile.patch” to cross check if all of code modification can be applied successfully

<Example>:

- It might encounter some failed during file patching, please fix it one by one and ensure each modification is success

<Example>:

checking file kernel/kernel-jammy-src/net/wireless/sme.c checking file kernel/kernel-jammy-src/net/wireless/trace.h checking file kernel/kernel-jammy-src/net/wireless/util.c checking file kernel/kernel-jammy-src/net/wireless/wext-compat.c checking file kernel/kernel-jammy-src/net/wireless/wext-sme.c checking file nvidia-oot/drivers/net/wireless/realtek/rtl8822ce/os_dep/linux/ioctl_cfg80211.c Hunk #9 FAILED at 5634. Hunk #10 FAILED at 7015. Hunk #11 succeeded at 11124 (offset -3 lines). 2 out of 11 hunks FAILED

- After correcting some failed items during patch process, please input “sudo patch -p1 -b < /path/to/your/patchfile.patch” to apply whole files

[AFE-R750]

$ sudo apt-get install qemu-user-static

$ cd ~/BSP

$ mkdir jetson_linux_risc

$ cd jetson_linux_risc

$ git config --global user.name "Your Name"

$ git config --global user.email "Your@example.com"

$ repo init -u https://AIM-Linux@dev.azure.com/AIM-Linux/risc_nvidia_jetson_36.4/_git/manifests -m afer750.xml

$ repo sync

$ /home/adv/BSP/jetson_linux_risc/kernel

$ git checkout origin/afer750-aiw173

- Build Kernel Image

Prebuild Image including wpa_supplicant & hostapd tool:

AIR-030_Jetpack_6.0_AIW-173customized_20251106.tar.gz

[AIR-030]

$ cd /home/adv/BSP/jetson_linux_risc

$ sudo ./scripts/build_release.sh -v <VERSION>

[AFE-R750]

$ cd /home/adv/BSP/jetson_linux_risc

$ sudo ./scripts/build_release.sh

- Build WLAN Driver

- Download the driver source code from git server

[AIR-030]

AIW_173_driver.tar.gz

$ cd / $ sudo git clone -b nvidia_jp_6_1 ssh://adv_ae@172.22.24.155:/volume1/git-server/AIW_173_driver

NOTE: the password for adv_ae is “123456”

[AFE-R750]

AIW_173_driver_AFE-R750.tar.gz

$ cd / $ sudo git clone ssh://adv_ae@172.22.24.155:/volume1/git-server/AIW_173_driver_AFE-R750

NOTE: the password for adv_ae is “123456”

- Copy header files to kernel

AIR-030’s folder name is “AIW_173_driver”
AFE-R750’s folder name is “AIW_173_driver_AFE-R750”

$ cd /AIW_173_driver_XXX /AIO/build $ sudo cp -r ../drivers/core_tech_modules/include/linux/mhi.h <kernel_path>/include/linux/ $ sudo cp -r ../drivers/core_tech_modules/include/trace/events/qrtr.h <kernel_path>/include/trace/events/ $ sudo cp -r ../drivers/core_tech_modules/inc/cnss2.h <kernel_path>/include/net/ $ sudo cp -r ../drivers/core_tech_modules/inc/cnss_nl.h <kernel_path>/include/net/ $ sudo cp -r ../drivers/core_tech_modules/inc/cnss_utils.h <kernel_path>/include/net/ $ sudo cp -r ../drivers/core_tech_modules/inc/cnss_prealloc.h <kernel_path>/include/net/

For example, <kernel-path>: /home/adv/BSP/jetson_linux_risc/kernel/kernel_out/kernel/kernel-jammy-src

- Build driver

$ cd /AIW_173_driver_XXX /AIO/build $ sudo make KERNELARCH=arm64 ARCH=arm64 KERNELPATH=/home/adv/BSP/jetson_linux_risc/kernel/kernel_out/kernel/kernel-jammy-src/ KERNEL_SRC=/home/adv/BSP/jetson_linux_risc/kernel/kernel_out/kernel/kernel-jammy-src/ CROSS_COMPILE=/opt/aarch64--glibc--stable-2022.08-1/bin/aarch64-buildroot-linux-gnu- CONFIG_FULL_CORE_BUILD=y CONFIG_PERF_BUILD=y drivers

- Get wlan_cnss_core_pcie.ko, wlan.ko from specific folder ( /AIW_173_driver_XXX /AIO/rootfs-tr-f10.build /lib/modules)

- Flash Image

[AIR-030]

- Enter to recovery mode on AIR-030
Switch the RESET_RECOVERY_1 SW-2 to ON and press the power button, the AIR-030 will boot into recovery mode

- Depressed your Image files from Docker

$ sudo tar -zxvf air030_image_*.tar.gz
$ cd Linux_for_Tegra

- Flash emmc/nvme

#emmc

$ sudo ./flash.sh jetson-agx-orin-devkit mmcblk0p1

#nvme

$ sudo ./tools/kernel_flash/l4t_initrd_flash.sh --external-device nvme0n1p1 -p "-c ./bootloader/generic/cfg/flash_t234_qspi.xml" -c ./tools/kernel_flash/flash_l4t_t234_nvme.xml --showlogs --network usb0 jetson-agx-orin-devkit nvme0n1p1

[AFE-R750]

- HW setting recover mode
Change the recover mode "2" to ON and press power button.

Connect a linux OS host wired AFE-R750 with Flash USB cable.
In the Host PC, make sure the AFE-R750 (NVidia Corp) has detectd.

[ubuntu@linux 11:55:43 ~]$ lsusb

Bus 001 Device 024: ID 0955:7223 NVidia Corp.

2.Extracted tarball file

$ sudo tar -zxvf afer750_image_XXXXXX.tar.gz

$ cd Linux_for_Tegra

3.Flash image to eMMC (AGX ORIN MODULE)

$ sudo ./flash.sh jetson-agx-orin-devkit mmcblk0p1

- Build hostapd and wpa_supplicant
(Local Device build only)

- Install related tool packages:

$ sudo apt install libssl-dev libdbus-1-dev flex bison libnl-3-dev libnl-genl-3-dev libnl-route-3-dev

- Copy whole AIW_173_Driver package from Docker to your local device

- Build hostapd

$ cd /home/adv/BSP/AIW_173_driver_XXX /AIO/apps/hostap/hostapd $ sudo make clean $ sudo cp defconfig .config $ make KERNELARCH=arm64 ARCH=arm64

- Build wpa_supplicant

$ cd /home/adv/BSP/AIW_173_driver_XXX/AIO/apps/hostap/wpa_supplicant $ sudo make clean $ sudo cp defconfig .config $ make KERNELARCH=arm64 ARCH=arm64

- Rename original “wpa_supplicant” tool and replace your rebuild version under specific path (/sbin)

- Confirm “wpa_supplicant” version if is 2.11 version and reboot your system

- Install Driver

Copy firmware files to device: $sudo cp -rf /AIW-173_driver_XXX /AIO/rootfs-tr-f10.build/lib/firmware/WLAN-firmware/* /lib/firmware/ $sudo cp -f /AIW-173_driver_XXX /AIO/ rootfs-tr-f10.build/lib /firmware/BT_firmware/usb/* /lib/firmware/qca Insmod driver: $ sudo modprobe cfg80211 $ sudo insmod <path to .ko>/wlan_cnss_core_pcie.ko $ sudo insmod <path to .ko>/wlan.ko country_code=US

- Check Device After installed driver

# ifconfig -a # ifconfig wlan0 up The result should like below: # iw wlan0 scan You will get scan result in the screen.

<Appendix>

Step 1

Establish 6Ghz STA mode by wpa_supplicant utility without using NetworkManager

- sudo lsmod | grep wlan

- sudo iw list | grep -A20 Frequencies

Step 2

Confirm your wifi interface name by specific command

- sudo iw dev

Step 3

Before running wpa_supplicant, please stop following system network service to avoid service conflict

•	Stop NetworkManager service

- sudo systemctl stop NetworkManager.service

•	Stop wpa_supplicant service

- sudo systemctl stop wpa_supplicant.service

Step 4

Copy WPA3 config file into your system and rewrite ssid/psk which you would like to connect WiFi AP’s SSID& Password

Step 5

clean remaining service before executing wpa_supplicant

- sudo killall wpa_supplicant

Step 6

Enter following command to run wpa_supplicant with your configuration file

- sudo ./wpa_supplicant -d -Dnl80211 -i wlp1s0 -c wpaXXX.conf &

Note: add “&” symbol in the end of command which will execute in background system.

For debugging, please remove it for further log tracking

Step 7

Request DHCP assignment to WiFI AP and confirm if ip has assigned successfully

- sudo dhclient wlp1s0

- sudo ifconfig wlp1s0

Note: please open another terminal to assign DHCP if wpa_supplicant is not running in background

Step 9

Confirm the connection is established & do ping testing

- sudo iw dev wlp1s0 link

- ping 192.168.2.21 -I wlp1s0

Step 10

We also verify 5Ghz connection with WPA3 encryption and it also work fine