AIW FAQ

How to bring up AIW-173 Wifi feature with RSB-3720

| Category | AIW | Date | 2025/11/27 |
| --- | --- | --- | --- |
| Keyword | AIW-173(WiFi 7),RSB-3720(Yocto 4.0 LTS) with linux kernel 5.15.52 |  |  |

Catergory:

1.	Check Device	2

2.	Build Environment on Host	2

3.	Getting Linux Code and Setting Environment	2

4.	Build Kernel Image and build Tool-Chain	5

5.	Build WLAN Driver and network tool(hostapd and wpa_supplicant)	5

6.	Flash Image	10

7.	Install Driver	11

8.	Check Device After installed driver	11

Appendix:	12

- Check Device

Before driver installed, you will see the device as below by “lspci”.

$ lspci 01:00.0 Network controller: Qualcomm Technologies, Inc WCN785x Wi-Fi 7(802.11be) 320MHz 2x2 [FastConnect 7800] (rev 01)

- Build Environment on Host

[RSB-3720]

https://ess-wiki.advantech.com.tw/view/IoTGateway/BSP/Linux/iMX8/Yocto_LBVD_User_Guide

- Getting Linux Code and Setting Environment

[RSB-3720]

$ repo init -u git://github.com/ADVANTECH-Corp/adv-arm-yocto-bsp.git -b imx-linux-kirkstone -m imx8LBVC0047.xml

$ repo sync

$ MACHINE=imx8mprsb3720a1 UBOOT_CONFIG=6G DISTRO=fsl-imx-xwayland source imx-setup-release.sh -b build_x11

$ bitbake virtual/kernel

$ mkdir -p ~/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/linux-imx/

Move below file in the “~/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/linux-imx/”

$ mv fragment.cfg ~/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/linux-imx/

$ nano ~/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/linux-imx_5.15.bbappend

Add content below at the end of this file.

SRC_URI+= "file://fragment.cfg"

do_configure:append () {

cat ${WORKDIR}/*.cfg >> ${B}/.config

}

For build hostapd and wpa_supplicant you will need add below tool in the local config

$ cd /home/adv/adv-release-bsp/build_x11/conf

$ vi local.conf

Add below content at the end of “local.conf”

TOOLCHAIN_TARGET_TASK:append = " libnl libnl-dev libnl-router dbus dbus-dev dbus-glib dbus-glib-dev openssl openssl-dev"

Rebuild kernel

$ bitbake -c cleansstate virtual/kernel

$ bitbake virtual/kernel

Check the .config file

$ cat /home/adv/adv-release-bsp/build_x11/tmp/work/imx8mprsb3720a1-poky-linux/linux-imx/5.15.52+gitAUTOINC+b630fd0dbf-r0/build/.config

Make the kernel folder can build driver

Copy the .config file to work-share folder

$ cp

/home/adv/adv-release-bsp/build_x11/tmp/work/imx8mprsb3720a1-poky-linux/linux-imx/5.15.52+gitAUTOINC+b630fd0dbf-r0/build/.config /home/adv/adv-release-bsp/build_x11/tmp/work-shared/imx8mprsb3720a1/kernel-source

$ cd /home/adv/adv-release-bsp/build_x11/tmp/work-shared/imx8mprsb3720a1/kernel-source

Note. You need have sdk to do below command. You can reference next chapter.

$ source /opt/fsl-imx-xwayland/5.15-kirkstone/environment-setup-armv8a-poky-linux

$ make ARCH=arm64 CROSS_COMPILE=aarch64-poky-linux- oldconfig

$ make ARCH=arm64 CROSS_COMPILE=aarch64-poky-linux- prepare

$ make ARCH=arm64 CROSS_COMPILE=aarch64-poky-linux- modules_prepare

- Build Kernel Image and build Tool-Chain

[RSB-3720]

$ cd ~/adv-release-bsp/build_x11/tmp/work/armv8a-poky-linux/nnshark/2021.10.imx-r0/git

$ git config submodule.common.url https://github.com/GStreamer/common.git

$ git submodule update

$ cd ~/adv-release-bsp/build_x11

$ bitbake imx-image-full

$ bitbake meta-toolchain

The below installer will be located in the directory "./tmp/deploy/sdk".

fsl-imx-wayland-glibc-x86_64-meta-toolchain-armv8a-imx8mp-lpddr4-evk-toolchain-${KERNEL}-scarthgap.sh

You need install the tool chain and source the environment

The full image will in the “./tmp/deploy/img”

- Build WLAN Driver and network tool(hostapd and wpa_supplicant)

5.1 Build WLAN Driver

- Download the driver source code from git server

[RSB-3720]

AIW_173_driver.tar.gz

$ cd / $ git clone ssh://adv_ae@172.22.24.155:/volume1/git-server/AIW_173_driver

NOTE: the password for adv_ae is “123456”

- Copy header files to kernel

RSB-3720’s folder name is “AIW_173_driver”

$ cd /AIW_173_driver_XXX /AIO/build $ sudo cp -r ../drivers/core_tech_modules/include/linux/mhi.h <kernel_path>/include/linux/ $ sudo cp -r ../drivers/core_tech_modules/include/trace/events/qrtr.h <kernel_path>/include/trace/events/ $ sudo cp -r ../drivers/core_tech_modules/inc/cnss2.h <kernel_path>/include/net/ $ sudo cp -r ../drivers/core_tech_modules/inc/cnss_nl.h <kernel_path>/include/net/ $ sudo cp -r ../drivers/core_tech_modules/inc/cnss_utils.h <kernel_path>/include/net/ $ sudo cp -r ../drivers/core_tech_modules/inc/cnss_prealloc.h <kernel_path>/include/net/

- Patch the driver

- $ cd /AIW_173_driver

- $ git add AIO

- $ git commit -m "Add AIO drivers baseline"

- $ git apply --3way 0001-add-some-modified-files.patch

You will see a lot of conflict as below
You can use below command to check which file you should modified

$ git diff --stat

You need manual modify all conflict and use git add the modified file to remove the git diff –stat

For example, when you modify the conflict file and you use git diff -stat, you will see the below information.

$ git add AIO/drivers/qca-wifi-host-cmn/hal/wifi3.0/kiwi/hal_kiwi.c
You will see the hal_kiwi.c is disappear

You will need modify all conflict file and use git diff –stat to check all file is modified.

- Build driver

$ sudo apt-get install build-essential libncurses5-dev flex bison libnl-3-dev $ sudo apt-get -y install kernel-package openssl $ sudo apt-get install libssl-dev $ sudo apt-get install libelf-dev $ sudo apt-get install -y libnl-genl-3-dev $ sudo apt-get install libdbus-1-dev $ sudo apt-get install git $ sudo apt install xterm tmux screen -y $ vim /AIW_173_driver/AIO/build/scripts/tr-f10/config.tr-f10 Modify below information in the config.tr-f10 export KERNELPATH=/home/adv/adv-release-bsp/build_x11/tmp/work-shared/imx8mprsb3720a1/kernel-source export KERNELARCH=arm64 export TOOLPREFIX=/opt/fsl-imx-xwayland/5.15-kirkstone/sysroots/x86_64-pokysdk-linux/usr/bin/aarch64-poky-linux/aarch64-poky-linux- $ cp /home/adv/adv-release-bsp/build_x11/tmp/work/imx8mprsb3720a1-poky-linux/linux-imx/5.15.52+gitAUTOINC+b630fd0dbf-r0/build/.config /home/adv/adv-release-bsp/build_x11/tmp/work-shared/imx8mprsb3720a1/kernel-source - $ cd /home/adv/adv-release-bsp/build_x11/tmp/work-shared/imx8mprsb3720a1/kernel-source - $ source /opt/fsl-imx-xwayland/5.15-kirkstone/environment-setup-armv8a-poky-linux - $ make ARCH=arm64 CROSS_COMPILE=aarch64-poky-linux- oldconfig - $ make ARCH=arm64 CROSS_COMPILE=aarch64-poky-linux- prepare - $ make ARCH=arm64 CROSS_COMPILE=aarch64-poky-linux- modules_prepare - $ cd /AIW_173_driver_XXX /AIO/build - $ sudo make CONFIG_FULL_CORE_BUILD=y CONFIG_PERF_BUILD=y drivers

- Get wlan_cnss_core_pcie.ko, wlan.ko from specific folder ( /AIW_173_driver_XXX /AIO/rootfs-tr-f10.build /lib/modules)

- The FW will in the specific folder

(/AIW_173_driver/AIO/rootfs-tr-f10.build/lib/firmware/WLAN-firmware)

  - Build hostapd and wpa_supplicant

- For build hostapd and wpa_supplicant you will need add below tool in the local config :

$ cd /home/adv/adv-release-bsp/build_x11/conf

$ vi local.conf

Add below content at the end of “local.conf”

- TOOLCHAIN_TARGET_TASK:append = " libnl libnl-dev dbus dbus-dev dbus-glib dbus-glib-dev openssl openssl-dev"

- IMAGE_INSTALL:append="dnsmasq linux-firmware-qca"

- Build hostapd

$ cd /AIW_173_driver /AIO/apps/hostap/hostapd $ sudo make clean $ make CROSS_COMPILE="aarch64-poky-linux-" CONFIG_FULL_CORE_BUILD=y CONFIG_PERF_BUILD=y

- Build wpa_supplicant

$ cd /AIW_173_driver/AIO/apps/hostap/wpa_supplicant $ sudo make clean $ make CROSS_COMPILE="aarch64-poky-linux-"CONFIG_FULL_CORE_BUILD=y CONFIG_PERF_BUILD=y

After built hostapd and wpa_supplicant, you need copy it to the SD card and boot up the RSB-3720. You need confirm “wpa_supplicant” version if is 2.11 version

- Flash Image

[RSB-3720]

Flash image to SD card

OS image path “/home/adv/adv-release-bsp/build_x11/tmp/deploy/images/imx8mprsb3720a1”

.wic.zst if full OS image

You need copy it to your system.

$ maker p ~/adv-release-bsp/Temp/SD_img

$ cp imx-image-full-imx8mprsb3720a1-20250916062435.rootfs.wic.zst ~/adv-release-bsp/Temp/SD_img

Change to your real system(Not in the docker)

- $ cd ~/Downloads/adv-release-bsp/Temp/SD_img

- $ zstd -d imx-image-full-imx8mprsb3720a1-xxxx.rootfs.wic.zst (Example: imx-image-full-imx8mprsb3720a1-20250916062435.rootfs.wic.zst)

- $ sudo dd if=imx-image-full-imx8mprsb3720a1-xxxx.rootfs.wic of=/dev/sdb bs=4M status=progress conv=fsync

- Install Driver

Copy firmware files to device: $sudo cp -rf /AIW-173_driver /AIO/rootfs-tr-f10.build/lib/firmware/WLAN-firmware/* /lib/firmware/ $sudo cp -f /AIW-173_driver /AIO/ rootfs-tr-f10.build/lib /firmware/BT_firmware/usb/* /lib/firmware/qca Insmod driver: $ sudo modprobe cfg80211 $ sudo insmod <path to .ko>/wlan_cnss_core_pcie.ko $ sudo insmod <path to .ko>/wlan.ko country_code=US

- Check Device After installed driver

# ifconfig -a # ifconfig wlan0 up The result should like below: # iw wlan0 scan You will get scan result in the screen.

Appendix:

wpa_supplicant test

2G wpa_supplicant config sample

5G wpa_supplicant config sample

6G wpa_supplicant config sample

Hostapd and iperf test

2G config

2G hostapd uplink

2G hostapd downlink

5G

5G config

5G hostapd uplink

5G hostapd downlink

6G

6G hostapd uplink

6G Downlink