AIW FAQ

How to bring up AIW-173 Wifi feature with RSB-3720

| Category | AIW | Date | 2025/12/9 |
| --- | --- | --- | --- |
| Keyword | AIW-173(WiFi 7),RSB-3720(Yocto 5.2 LTS) with linux kernel 6.6.23) |  |  |

Catergory:

1.	Check Device	2

2.	Build Environment on Host	2

3.	Getting Linux Code and Setting Environment	2

4.	Build Kernel Image and build Tool-Chain	5

5.	Build WLAN Driver and network tool(hostapd and wpa_supplicant)	5

6.	Flash Image	10

7.	Install Driver	11

8.	Check Device After installed driver	11

Appendix:	12

- Check Device

Before driver installed, you will see the device as below by “lspci”.

$ lspci 01:00.0 Network controller: Qualcomm Technologies, Inc WCN785x Wi-Fi 7(802.11be) 320MHz 2x2 [FastConnect 7800] (rev 01)

- Build Environment on Host

[RSB-3720]

https://ess-wiki.advantech.com.tw/view/IoTGateway/BSP/Linux/iMX8/Yocto_LBVF_User_Guide

- Getting Linux Code and Setting Environment

[RSB-3720]

$ repo init -u git://github.com/ADVANTECH-Corp/adv-arm-yocto-bsp.git -b imx-linux-kirkstone -m imx8LBVF0071.xml

$ repo sync

$ MACHINE=imx8mprsb3720a2 UBOOT_CONFIG=6G DISTRO=fsl-imx-xwayland source imx-setup-release.sh -b build_x11

$ bitbake virtual/kernel

$ mkdir -p ~/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/files

Move below file in the “/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/files”

$ mkdir -p ~/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/linux-imx/

Move below file in the “~/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/linux-imx/”

$ mv fragment.cfg ~/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/linux-imx/

$ nano ~/adv-release-bsp/sources/meta-advantech/meta-fsl-imx/recipes-kernel/linux/linux-imx_6.6.bbappend

Add content below at the end of this file.

FILESEXTRAPATHS:prepend := "${THISDIR}/files:"

SRC_URI += "file://0001-kernel-mods-for-6.6.23.patch"

KCONFIG_MODE = "merge"

SRC_URI += "file://fragment.cfg"

do_configure:append () {

cat ${WORKDIR}/*.cfg >> ${B}/.config

}

For build hostapd and wpa_supplicant you will need add below tool in the local config

$ cd /home/adv/adv-release-bsp/build_x11/conf

$ vi local.conf

Add below content at the end of “local.conf”

TOOLCHAIN_TARGET_TASK:append = " libnl libnl-dev libnl-router dbus dbus-dev dbus-glib dbus-glib-dev openssl openssl-dev"

Rebuild kernel

$ bitbake -c cleansstate virtual/kernel

$ bitbake virtual/kernel

Check the .config file

$ cat /home/adv/adv-release-bsp/build_x11/tmp/work/imx8mprsb3720a2-poky-linux/linux-imx/6.6.23+git/build /.config

Copy the .config file to work-share folder

$ cp

/home/adv/adv-release-bsp/build_x11/tmp/work/imx8mprsb3720a2-poky-linux/linux-imx/6.6.23+git/build /.config /home/adv/adv-release-bsp/build_x11/tmp/work-shared/imx8mprsb3720a2/kernel-source

$ cd /home/adv/adv-release-bsp/build_x11/tmp/work-shared/imx8mprsb3720a2/kernel-source

Note. You need have sdk to do below command. You can reference next chapter.

$ source /opt/fsl-imx-xwayland/5.15-kirkstone/environment-setup-armv8a-poky-linux

$ make ARCH=arm64 CROSS_COMPILE=aarch64-poky-linux- oldconfig

$ make ARCH=arm64 CROSS_COMPILE=aarch64-poky-linux- prepare

$ make ARCH=arm64 CROSS_COMPILE=aarch64-poky-linux- modules_prepare

$ make ARCH=arm64 CROSS_COMPILE=aarch64-poky-linux- modules

- Build Kernel Image and build Tool-Chain

[RSB-3720]

$ vi /home/adv/adv-release-bsp/sources/meta-imx/meta-imx-ml/recipes-nnstreamer/nnshark/nnshark_2023.11.imx.bb

Modify

NNSHARK_SRC ?= "gitsm://github.com/nxp-imx/nnshark.git;protocol=https"

to

NNSHARK_SRC ?= "git://github.com/nxp-imx/nnshark.git;protocol=https"

add below content in the nnshardk_2023.11.imx.bb

do_configure[network]="1"

do_configure:prepend() {

cd ${S}

echo ">>> Fixing GStreamer common submodule URL"

if grep -q "anongit.freedesktop.org" .gitmodules; then

sed -i 's#git://anongit.freedesktop.org/gstreamer/common#https://gitlab.freedesktop.org/gstreamer/common.git#g' .gitmodules || true

fi

git submodule sync --recursive || true

git submodule update --init --recursive || true

echo ">>> Submodule fix done."

}

$ vi ~/adv-release-bsp/sources/meta-imx/meta-imx-ml/recipes-libraries/onnxruntime/onnxruntime_1.17.1.bb

Add below function as below

DEPENDS = "libpng zlibi libeigen"

EXTRA_OECMAKE += "\

-DFETCHCONTENT_FULLY_DISCONNECTED=OFF \

-DCMAKE_BUILD_TYPE=RelWithDebInfo \

-Donnxruntime_BUILD_UNIT_TESTS=ON \

-Donnxruntime_USE_PREINSTALLED_EIGEN=ON\

-Deigen_SOURCE_PATH=${STAGING_INCDIR}/eigen3\

"

$ cd ~/adv-release-bsp/build_x11

$ bitbake imx-image-full

$ bitbake meta-toolchain

The below installer will be located in the directory "./tmp/deploy/sdk".

fsl-imx-wayland-glibc-x86_64-meta-toolchain-armv8a-imx8mp-lpddr4-evk-toolchain-${KERNEL}-scarthgap.sh

You need install the tool chain and source the environment

The full image will in the “./tmp/deploy/img”

- Build WLAN Driver and network tool(hostapd and wpa_supplicant)

5.1 Build WLAN Driver

- Download the driver source code from git server

[RSB-3720]

AIW_173_driver.tar.gz

$ cd / $ sudo git clone ssh://adv_ae@172.22.24.155:/volume1/git-server/AIW_173_driver $ sudo mv /AIW_173_driver /173 $ cd /173/

NOTE: the password for adv_ae is “123456”

- Copy header files to kernel

RSB-3720’s folder name is “AIW_173_driver”

$ cd / 173 /AIO/build $ sudo cp -r ../drivers/core_tech_modules/include/linux/mhi.h <kernel_path>/include/linux/ $ sudo cp -r ../drivers/core_tech_modules/include/trace/events/qrtr.h <kernel_path>/include/trace/events/ $ sudo cp -r ../drivers/core_tech_modules/inc/cnss2.h <kernel_path>/include/net/ $ sudo cp -r ../drivers/core_tech_modules/inc/cnss_nl.h <kernel_path>/include/net/ $ sudo cp -r ../drivers/core_tech_modules/inc/cnss_utils.h <kernel_path>/include/net/ $ sudo cp -r ../drivers/core_tech_modules/inc/cnss_prealloc.h <kernel_path>/include/net/

- Build driver

$ sudo apt-get install build-essential libncurses5-dev flex bison libnl-3-dev $ sudo apt-get -y install kernel-package openssl $ sudo apt-get install libssl-dev $ sudo apt-get install libelf-dev $ sudo apt-get install -y libnl-genl-3-dev $ sudo apt-get install libdbus-1-dev $ sudo apt-get install git $ sudo apt install xterm tmux screen -y $ vim /173/AIO/build/scripts/tr-f10/config.tr-f10 Modify below information in the config.tr-f10 export KERNELPATH=/home/adv/adv-release-bsp/build_x11/tmp/work-shared/imx8mprsb3720a2/kernel-source export KERNELARCH=arm64 export TOOLPREFIX=/opt/fsl-imx-xwayland/6.6-scarthgap/sysroots/x86_64-pokysdk-linux/usr/bin/aarch64-poky-linux/aarch64-poky-linux- Then modify driver +++ b/AIO/drivers/qca-wifi-host-cmn/hif/src/ce/ce_main.c @@ -2904,7 +2904,7 @@ hif_send_head(struct hif_opaque_softc *hif_ctx, if (qdf_unlikely(!ce_hdl)) { hif_err("CE handle is null"); - return A_ERROR; + return (QDF_STATUS)A_ERROR; } QDF_NBUF_UPDATE_TX_PKT_COUNT(nbuf, QDF_NBUF_TX_PKT_HIF); diff --git a/AIO/drivers/qcacld-3.0/Kbuild b/AIO/drivers/qcacld-3.0/Kbuild index f0d0f4a..223e1c5 100644 --- a/AIO/drivers/qcacld-3.0/Kbuild +++ b/AIO/drivers/qcacld-3.0/Kbuild @@ -3256,6 +3256,8 @@ ccflags-y += $(INCS) cppflags-y += -DANI_OS_TYPE_ANDROID=6 \ -Wall\ -Werror\ + -Wno-error=enum-int-mismatch\ + -Wno-error=address\ -D__linux__ $ cd /AIW_173_driver_XXX /AIO/build $ sudo make CONFIG_FULL_CORE_BUILD=y CONFIG_PERF_BUILD=y drivers

- Get wlan_cnss_core_pcie.ko, wlan.ko from specific folder ( /AIW_173_driver_XXX /AIO/rootfs-tr-f10.build /lib/modules)

- The FW will in the specific folder

(/AIW_173_driver/AIO/rootfs-tr-f10.build/lib/firmware/WLAN-firmware)

  - Build hostapd and wpa_supplicant

- For build hostapd and wpa_supplicant you will need add below tool in the local config :

$ cd /home/adv/adv-release-bsp/build_x11/conf

$ vi local.conf

Add below content at the end of “local.conf”

- TOOLCHAIN_TARGET_TASK:append = " libnl libnl-dev dbus dbus-dev dbus-glib dbus-glib-dev openssl openssl-dev"

- IMAGE_INSTALL:append="dnsmasq linux-firmware-qca"

- Build hostapd

$ cd /AIW_173_driver /AIO/apps/hostap/hostapd $ sudo make clean $ make hostapd

- Build wpa_supplicant

$ cd /AIW_173_driver/AIO/apps/hostap/wpa_supplicant $ sudo make clean $ make wpa_supplicant

After built hostapd and wpa_supplicant, you need copy it to the SD card and boot up the RSB-3720. You need confirm “wpa_supplicant” version if is 2.11 version

- Flash Image

[RSB-3720]

Flash image to SD card

OS image path “/home/adv/adv-release-bsp/build_x11/tmp/deploy/images/imx8mprsb3720a2”

.wic.zst if full OS image

You need copy it to your system.

$ maker p ~/adv-release-bsp/Temp/SD_img

$ cp imx-image-full-imx8mprsb3720a2-20251209023921.rootfs.wic.zst ~/adv-release-bsp/Temp/SD_img

Change to your real system(Not in the docker)

- $ cd ~/Downloads/adv-release-bsp/Temp/SD_img

- $ zstd -d imx-image-full-imx8mprsb3720a1-xxxx.rootfs.wic.zst (Example: imx-image-full-imx8mprsb3720a2-20251209023921.rootfs.wic.zst)

- $ sudo dd if=imx-image-full-imx8mprsb3720a1-xxxx.rootfs.wic of=/dev/sdb bs=4M status=progress conv=fsync

- Install Driver

Copy firmware files to device: $sudo cp -rf /AIW-173_driver /AIO/rootfs-tr-f10.build/lib/firmware/WLAN-firmware/* /lib/firmware/ Insmod driver: $ sudo modprobe cfg80211 $ sudo insmod <path to .ko>/wlan_cnss_core_pcie.ko $ sudo insmod <path to .ko>/wlan.ko country_code=US

- Check Device After installed driver

# ifconfig -a # ifconfig wlan0 up The result should like below: # iw wlan0 scan You will get scan result in the screen.

Appendix:

wpa_supplicant test

When you use wpa_supplicant to connect AP, you need to get the IP by using “udhcpc” command.

2G wpa_supplicant config sample:

Command wpa_supplicant -i wlan0 -c wpa_supplicant_2G.conf

5G wpa_supplicant config sample

6G wpa_supplicant config sample

When you use hostapd, your system need “dnsmasq” as the DHCP server. You need stop the “connmon” and “dnsmasq” service then manual execute dnsmasq with dnsmasq.conf.

2G Hostapd

5G Hostapd

6G Hostapd